## 2. Verification Plan (Word-ready)

### 2.1 Scope

- DUT: axi2apb (32→32), default parameters.

- Excluded: axi2apb_64_32 (burst variant), axi_slice family (not instantiated).

- Verification method: UVM dual-agent (AXI master + APB slave), scoreboard, SVA, functional coverage, directed + constrained-random regression.

### 2.2 DUT Interface Summary

| Side | Signal Group | Direction (DUT) |
| --- | --- | --- |
| AXI | AW (ID/ADDR/LEN/SIZE/BURST/LOCK/CACHE/PROT/REGION/USER/QOS/VALID/READY) | slave |
| AXI | W (DATA/STRB/LAST/USER/VALID/READY) | slave |
| AXI | B (ID/RESP/USER/VALID/READY) | master |
| AXI | AR (same fields as AW) | slave |
| AXI | R (ID/DATA/RESP/LAST/USER/VALID/READY) | master |
| APB | PSEL, PENABLE, PWRITE, PADDR, PWDATA | master |
| APB | PRDATA, PREADY, PSLVERR | slave |
| Misc | ACLK, ARESETn, test_en_i | in |

### 2.3 DUT Behavioral Model (from RTL — this is your reference model spec)

Reset (ARESETn=0): CS<=IDLE, RDATA_Q<='0. Buffers cleared.

Write path:

- Master pushes AW into AW-FIFO (when AWREADY_o=~full). Same for W.

- In IDLE, FSM sees AWVALID & WVALID (FIFO outputs). Sets write_req=1, address=AWADDR[APB_ADDR_WIDTH-1:0]. PENABLE=1, PWRITE=1, PSEL=1, PWDATA=WDATA.

- If PREADY=1 in IDLE → SEND_B_RESP. Else → WAIT_W_PREADY (keeps write_req=1).

- In WAIT_W_PREADY, when PREADY=1 → SEND_B_RESP.

- In SEND_B_RESP: BVALID=1, BID=AWID, BUSER=AWUSER, BRESP=OKAY. When BREADY=1: pop AW-FIFO and W-FIFO (AWREADY=1, WREADY=1), NS=IDLE.

Read path:

- Master pushes AR into AR-FIFO.

- In IDLE, if ARVALID=1 (priority over AW): sets read_req=1, address=ARADDR[APB_ADDR_WIDTH-1:0], sample_RDATA=PREADY.

- If PREADY=1 in IDLE: RDATA_Q<=PRDATA next cycle, NS=DONE_SINGLE_RD. Else → WAIT_R_PREADY.

- In WAIT_R_PREADY: same read_req=1, when PREADY=1 → DONE_SINGLE_RD + RDATA_Q<=PRDATA.

- In DONE_SINGLE_RD: RVALID=1, RLAST=1, RDATA=RDATA_Q, RID=ARID, RUSER=ARUSER, RRESP=OKAY. When RREADY=1: NS=IDLE, ARREADY=1 (pop AR-FIFO).

Priority: In IDLE, ARVALID is checked first → read wins.

Response values: Always OKAY. PSLVERR ignored.

Burst: axi2apb does not implement burst address progression. AWLEN/ARLEN/AWSIZE/ARSIZE/AWBURST/ARBURST/WSTRB/WLAST are buffered but not used by the FSM.

### 2.4 Verification Strategy

- AXI Master Agent: drives AW, W, AR; monitors B, R. Reconstructs full write/read transactions.

- APB Slave Agent: responds to PSEL/PENABLE/PWRITE. Configurable PRDATA, PREADY delay, PSLVERR.

- Scoreboard (reference model):

  - AXI write → predict APB PADDR/PWRITE/PWDATA; expect BVALID with BID=AWID, BUSER=AWUSER, BRESP=OKAY.

  - AXI read → predict APB PADDR/PWRITE=0; expect RVALID with RDATA=PRDATA, RID=ARID, RUSER=ARUSER, RRESP=OKAY, RLAST=1.

  - Correlate via a transaction ID queue (TB-side).

- Assertions (SVA): listed in matrix below.

- Functional coverage: cross of txn type × PREADY latency × backpressure.

- Tests: see matrix.

### 2.5 Sign-off Criteria

| Metric | Target |
| --- | --- |
| Directed tests | 100% pass |
| Constrained-random | ≥ 500 transactions/seed × 5 seeds, 100% pass |
| Code coverage (DUT) | ≥ 95% |
| Functional coverage | 100% defined bins |
| Assertions | 0 failures |
| Lint | Clean or documented |
| Known limitations | Documented |