Final Verification Report: AXI4-to-APB Bridge UVM Verification

Oct 18, 2026

# Executive Summary

The axi2apb bridge passed all 17 tests with zero UVM errors, zero scoreboard mismatches and 100% functional coverage. A dual-agent UVM environment was built from scratch: an AXI master agent, an APB slave agent, a transaction-level scoreboard and functional coverage collectors.

The 17 tests are 14 directed and 3 constrained-random. Structural coverage gaps (FSM, condition, line) were analysed and documented as testbench limitations or RTL structure, not DUT bugs.

Author: Muhammad Sheharyar, submitted to Engr. Shahbaz Ahmad

Simulator: Synopsys VCS (L-2016.06) with UVM 1.2

Repository: AXI-2-APB_Bridge_UVM_Verification

# Design Under Verification (DUT)

The DUT is the PULP-platform axi2apb module, which converts AXI4 single-beat read and write requests into APB transfers. The RTL was verified as supplied and not modified.

Key RTL characteristics:

The FSM gives Read (AR) priority over Write (AW) in the IDLE state.

A write is accepted only when both AWVALID and WVALID are asserted.

Burst transactions are not supported (single-beat only).

PSLVERR is not used by the FSM; responses are always OKAY.

| State | Behaviour | Next state |
| --- | --- | --- |
| IDLE | Read first; write needs AW and W together | Any other state |
| WAIT_R_PREADY | PENABLE high, waits for PREADY | DONE_SINGLE_RD |
| WAIT_W_PREADY | PENABLE and PWRITE high, waits for PREADY | SEND_B_RESP |
| State | Behaviour | Next state |
| DONE_SINGLE_RD | RVALID and RLAST until RREADY | IDLE |
| SEND_B_RESP | BVALID until BREADY | IDLE |

# Repository Structure

The project follows the folder layout required by the specification.

| Folder | Contents |
| --- | --- |
| rtl/ | DUT: axi2apb.sv, the AW/AR/W/R/B buffers, axi_single_slice and fifo |
| tb/ | Interfaces (axi_if, apb_if), UVM package, tb_top (clock, reset, DUT instance) |
| uvm/ | axi/ and apb/ agents (driver, monitor, sequencer, agent); env/ with environment, scoreboard and coverage |
| sequences/ | AXI write, read, random and single-item sequences |
| tests/ | Base test, 14 directed tests, 3 random tests |
| sim/ | Makefile, build.flist, out/ (build artifacts and coverage database) |
| scripts/ | parse_log.py, which parses the UVM summary from logs |
| logs/, results/ | Simulation logs and regression summaries (generated) |
| docs/ | Verification plan and this final report |

# 4.UVM Environment Architecture

The environment uses standard UVM 1.2 methodology. Data flows from the AXI sequences through the AXI driver into the DUT, out to the APB side, and both monitors feed the scoreboard and the coverage collector through analysis ports.

AXI agent (active): drives the AW, W and AR channels and monitors B and R. The monitor reconstructs complete AXI transactions and sends them to the scoreboard.

APB agent (active): monitors APB requests (PSEL, PENABLE) and drives PREADY, PRDATA and PSLVERR using configurable wait states.

Scoreboard: keeps queues of AXI and APB transactions and matches them in order. It checks that address, data, ID and USER propagate correctly and that AXI responses are always OKAY.

Coverage collector: receives data from both monitors through uvm_analysis_imp_decl and samples two covergroups, cg_axi and cg_apb.

# 5. Verification Results

All 17 tests of the final regression completed with UVM_ERROR: 0 and UVM_FATAL: 0.

| Type | Test | Status |
| --- | --- | --- |
| Directed | reset_test | PASS |
| Directed | axi_write_ready_test | PASS |
| Directed | axi_read_ready_test | PASS |
| Directed | axi_write_wait_test | PASS |
| Directed | axi_read_wait_test | PASS |
| Directed | bready_delay_test | PASS |
| Directed | rready_delay_test | PASS |
| Directed | rd_wr_priority_test | PASS |
| Directed | bk2bk_read_test | PASS |
| Directed | bk2bk_write_test | PASS |
| Directed | mixed_traffic_test | PASS |
| Directed | id_user_test | PASS |
| Directed | pslverr_test | PASS |
| Directed | coverage_closure_test | PASS |
| Random | rand_test | PASS |
| Type | Test | Status |
| Random | rand_axi_test (100 txns) | PASS |
| Random | rand_mixed_test (100 txns) | PASS |

# 6. Coverage Analysis and Engineering Trade-offs

Functional coverage reached 100%, while structural coverage is lower for documented reasons.

| Metric | Result | Root cause |
| --- | --- | --- |
| Functional (GROUP) | 100.00% | All bins hit, including crosses of transaction type, APB wait states (0, 1, 5), BREADY/RREADY delays and data/address patterns, closed with coverage_closure_test |
| FSM | 60.00% | IDLE to SEND_B_RESP and IDLE to DONE_SINGLE_RD need a true zero-wait APB response, because the DUT checks PREADY inside IDLE |
| Condition (COND) | 45.45% | Parameterized dead code (fifo.sv tests DEPTH == 0 while DEPTH is fixed at 4) and hard-wired signals such as PSEL = 1'b1 |
| Line | ~80.00% | Unreachable FSM default cases and unused FIFO branches |

FSM trade-off. The APB driver was changed to blocking assignments to emulate a zerowait response combinationally. This raised FSM coverage to 80%, but it introduced deltacycle races between the testbench and the RTL and caused intermittent scoreboard failures on reads. The driver was reverted to a standard clocked driver to keep the environment stable. The missing transitions are documented as a testbench limitation, not a DUT bug.

# 7.Known Limitations and Documented RTL Behavior

Three behaviours of the supplied RTL were documented rather than assumed:

No burst support: the FSM does not progress addresses and handles single-beat transfers only. AWLEN and ARLEN are buffered but ignored.

WSTRB ignored: the APB generation logic does not gate writes based on WSTRB. PSLVERR ignored: the FSM does not use PSLVERR; BRESP and RRESP are hard-coded to OKAY.

# 8.Conclusion

The axi2apb bridge was thoroughly verified, and the target of 100% functional coverage was met. The environment confirms correct AXI-to-APB conversion, ID and USER propagation, wait-state handling and back-pressure behaviour.

Structural coverage gaps were analysed in depth. Choosing testbench stability over inflating FSM coverage was a deliberate engineering decision.

Sign-off status: complete.

# Appendix: How to Reproduce

- Go to the sim/ directory.

- Run make allclean to clear previous artifacts.

- Run make regression to run all 17 tests and write results/regression_summary.txt .

- Run make cov to generate the coverage dashboard in out/coverage_report/ .